# Master Blueprint: 4-DOF Wireless Gesture-Controlled Robot Arm

Phase 1: The High-Level Overview (Big Picture)

The data flows like a river through the system:

Motion Hand→ Transmitter ESP32→Wireless→ Receiver ESP32→Wired → Controller (TM4C) → Action (Servos)

Phase 2: The Transmitter (The Glove/Remote)

This unit reads your hand movements and creates the data packet.

Parts:

- ESP32 NodeMCU: The Brain.

- MPU9250: Accelerometer + Gyroscope (Communicates via I2C).1

- Power: 2x 18650 Li-ion Batteries (7.4V) connected to the VIN pin.

- Button: BOOT Button (GPIO 0) used for Claw control.

Connections:

- MPU VCC →  ESP 3.3V

- MPU GND →  ESP GND

- MPU SDA →  GPIO 21

- MPU SCL →  GPIO 22

Software Logic (Loop Flow):

- Read Sensors: Acquire raw Accelerometer and Gyroscope data from the MPU9250.

- Process (Madgwick): Feed the raw data into the Madgwick filter algorithm →  Output Roll, Pitch, and Yaw (Float values).

- Map: Convert the Float angles (e.g., -90 to +90 degrees) into the Servo Range (0 to 180) using the map() function.

- Check Claw:

  - If BOOT button is pressed →  Set Claw = 180 (Close).

  - Else →  Set Claw = 0 (Open).

- Pack: Stuff the mapped data into a struct (Total size: 4 Bytes).

- Send: Fire the data wirelessly using esp_now_send()!

Phase 3: The Receiver Bridge (The Postman)

Its job is strictly that of a postman. Take the packet and pass it forward.

Parts:

- ESP32 NodeMCU: Acts only as a WiFi receiver.

- Power: Powered via the a shared battery rail.

Connections (Crucial Step):

- ESP32 Tx2 (GPIO 17) → TM4C Rx1 (PB0).

- ESP32 GND →  TM4C GND (Common Ground is mandatory!).

Software Logic:

- Listen: The OnDataRecv() callback function waits for incoming data.

- Receive: As soon as a packet arrives, copy it into the local struct.

- Re-Package: Create an "Envelope" or Frame for the TM4C:

  - Header (0xFF)

  - Base Byte

  - Shoulder Byte

  - Elbow Byte

  - Claw Byte

  - Footer (0xFE)

- Forward: Send the frame using Serial2.write().

Phase 4: The Robot Controller (The Muscle Brain)

The real hard work happens here: Data parsing and Motor synchronization.

Parts:

- Tiva C (TM4C123GH6PM): Main Controller.

- 4x Servo Motors: (Base, Shoulder, Elbow, Claw).

- Power: 3x 18650 Batteries (11.1V).

- Voltage Regulator: XL4015 Buck Converter (Step down 11.1V to 6V).

Connections:

- UART: PB0 (Rx1) receiving data from the ESP32.

- PWM Pins:

  - Base →  PB6 (M0PWM0)

  - Shoulder →  PB4 (M0PWM2)

  - Elbow →  PE4 (M0PWM4)

  - Claw →  PC4 (M0PWM6) (Note: Check PinMux; if PC4 is tricky/locked, use another Generator).

Power Wiring :

- Batteries →  XL4015 Input.

- XL4015 Output (Set to 6V) →  Servo VCC pins (Red wires).

- Servo GND (Brown wires) →  XL4015 Output GND AND TM4C GND.

- Servo Signal (Orange wires) →  TM4C PWM Pins.

Software Logic:

- Init: Setup UART (115200 baud) + Setup PWM (50Hz frequency, Sync enabled).

- UART Interrupt: When data arrives, perform checks:

  - "Is the first byte 0xFF?" →  Yes.

  - "Store the next 4 bytes."

  - "Is the last byte 0xFE?" →  Yes.

- Action: If the packet is valid:

  - Convert received values (0-180) into PWM Duty Cycle (High Time counts).

  - Formula: Duty = (Angle * Scaling) + Min_Pulse.

  - Update CMPA registers.

  - Trigger PWM_SYNC so all motors move simultaneously.

Phase 5: The Part Flow (Bill of Materials)

Electronics:

- 2x ESP32 NodeMCU.

- 1x MPU9250 Module.

- 1x TM4C123 Tiva C Launchpad.

- 4x Servo Motors (Use MG996R for Base/Shoulder, SG90 for Claw).

- 1x XL4015 Buck Converter Module.

Power:

- 5x 18650 Li-ion Cells (2 for Tx, 3 for Rx).

- Battery Holders (2-slot and 3-slot).

- Jumper Wires (Female-Female, Male-Female).

Mechanical:

- 3D Printed Robot Arm parts (or Acrylic kit).

Final Checklist for "MPS Wale Bhai" Setup:

- Calibrate First: Place the MPU9250 flat on a table and calibrate it (using the 'c' command feature in your code) before use.

- Buck Converter Tuning: Connect the XL4015 to the battery and turn the screw to set the output voltage to 6V before connecting the Servos. Otherwise, the servos will go "Kaboom".

- Common Ground: I cannot stress this enough—The ESP32, TM4C, and Buck Converter grounds must all be connected together.